EE2016 LAB EXPERIMENT 3 – ARM BASICS

EE20B056

TASK 1 – FACTORIAL:

Compute the factorial of a given number using ARM processor through assembly programming

CODE:

AREA FACTORIAL,CODE,READONLY

LDR	R0,NUM

LDR	R1,NUM

fact

SUBS	R0,R0,#0

BEQ	one

SUBS	R0,R0,#1

BEQ	one

MUL	R2,R1,R0

MOV	R1,R2

B fact

one

MOV	R2,#1

b	one

NUM	DCW	&5

END

LOGIC:	Given number is put in R0 and R1, firstly we check if given number is 1 if true, we jump to ‘one’ label meaning the factorial is 1, if given number is not 1 , we subtract the value in R0 by 1 and multiply with the value in R1 and put it in R2, and we copy this value back to R1 and proceed the same way. For a given value the process is shown below

[R0,R1,R2] : [5,-,-]  [5,5,-]  [4,5,-]  [4,5,20]  [4,20,20]  [3,20,20]  [3,20,60]  [3,60,60]  [2,60,60]  [2,60,120]  [2,120,120]  [1,120,120]  [1,120,1]

TASK 2 – 16-bit HALFWORD:

Combine the low four bits of each of the four consecutive bytes beginning at LIST into one 16-bit halfword. The value at LIST goes into the most signicant nibble of the result. Store the result in the 32-bit variable RESULT.

CODE:

AREA Program ,CODE,READONLY

LDR R0, list ; four consecutive bytes as defined below

LDR	R5,	mask ; 4 LSB bits alone are kept as 1s to be used as mask

LDR	R10, Result ; the address being labelled as result is copied to this register

MOV	R1,R0

AND	R1,R5 ; the first 4 lsb bits alone are kept

MOV	R1,R1,LSL	#4 ; the four bits are shifted to left,

MOV	R0,R0,LSR	#8 ; eight bits here are shifted to the right so that the  MSB half byte is removed

MOV	R2,R0

AND	R2,R5 – mask the other bits than the lower half byte of the next byte alone

ORR	R1,R2

MOV	R1,R1,LSL	#4

MOV	R0,R0,LSR	#8

MOV	R2,R0

AND	R2,R5

ORR	R1,R2

MOV	R1,R1,LSL	#4

MOV	R0,R0,LSR	#8

MOV	R2,R0

AND	R2,R5

ORR	R1,R2

STR	R1,[R10]

done	b	done

list

dcb	&0c

dcb	&02

dcb	&06

dcb	&09

mask	dcd	&0000000F

Result	dcd	&40000000

END

LOGIC:

R0 – LIST – 4 bytes sequenced as a single 32 bit

R1 – output - LSBs of the given 4 bytes in list

R2 – used to take the lower byte one by one by masking with R0 with R5

R5 – 0x0000000F – mask, when masked with this produces 4 bit LSBs alone

The above registers are used as mentioned in the comments in the code

TASK 3 – ODD OR EVEN:

Given a 32-bit number, identify whether it is an even or odd. (You implementation should not involve division).

CODE:

AREA program,CODE,READONLY

ENTRY

LDR R1,NUM

MOVS	R1,R1,LSR #1

bcs odd

even

MOV	R0,#0

b	even

odd

MOV	R0,#1

b	odd

swi	&11

NUM DCD  0x00000004

align

END

LOGIC:

Given number is taken in R1

R1 is logically shifted to right , if it is odd carry is set, if it is even carry is not set – in both cases it jumps to either of the loops corresponding to the case.